EHS COMMUNITY Confined Space Entry Permit Per 29 CFR 1910.146 — Permit-Required Confined Spaces

Permit #

⚠ PERMIT-REQUIRED CONFINED SPACE ENTRY. This permit must be completed and signed by the Entry Supervisor before any entrant enters the space. The permit is valid only for the date and duration listed. Cancel immediately if conditions change or the job scope changes.

SECTION 1 — SPACE & JOB IDENTIFICATION

| Space Name / Location | Space Description / ID | Purpose of Entry |
| --- | --- | --- |

| Permit Date | Valid From (Time) | Permit Expires (Time) | Maximum # Entrants |
| --- | --- | --- | --- |

SECTION 2 — HAZARDS PRESENT IN THIS SPACE

| Potential Hazard | Present? (Y/N) | Control Measure |
| --- | --- | --- |
| Oxygen-deficient atmosphere (<19.5%) |  |  |
| Oxygen-enriched atmosphere (>23.5%) |  |  |
| Flammable gas / vapor (>10% LEL) |  |  |
| Toxic air contaminant (above PEL) |  |  |
| Engulfment / flooding hazard |  |  |
| Internal configuration / entrapment |  |  |
| Mechanical / energized equipment |  |  |
| Thermal (heat / cold stress) |  |  |
| Other: _______________________ |  |  |

SECTION 3 — ATMOSPHERIC TESTING LOG

Before entry, test in this order: oxygen, then combustible gases and vapors, then toxic gases and vapors (29 CFR 1910.146(d)(5)(iii)). During entry, test or monitor as often as needed to confirm acceptable conditions are maintained (d)(5)(ii); monitor continuously where the space cannot be isolated (d)(5)(i). Continuous monitoring, and testing at several levels where the atmosphere may be stratified (Appendix B to 1910.146), are good practice. Column limits are US federal OSHA values; Cal/OSHA's are lower (8 CCR 5155: CO 25 ppm and H₂S 10 ppm 8-hour PELs) and Canadian provinces set their own, so use the limits for your jurisdiction.

| Time | Tested By | Instrument / Cal. Date | O₂ % (19.5–23.5%) | LEL % (<10%) | CO ppm (US PEL: 50, 8-hr TWA) | H₂S ppm (US ceiling: 20) | Other | Safe to Enter? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

SECTION 4 — ISOLATION & ENERGY CONTROL

| LOTO Procedure # / Equipment Isolated | Lines Blanked / Blinded | Ventilation Provided (Type / CFM) |
| --- | --- | --- |

SECTION 5 — REQUIRED PPE

| Respiratory Protection | Retrieval / Rescue Equipment | Other PPE Required |
| --- | --- | --- |

SECTION 6 — PERSONNEL

| Role | Name (Print) | Signature | Time In | Time Out |
| --- | --- | --- | --- | --- |
| Entry Supervisor |  |  | — | — |
| Attendant (Outside) |  |  | — | — |
| Authorized Entrant 1 |  |  |  |  |
| Authorized Entrant 2 |  |  |  |  |
| Rescue Service Contact |  | — | — | — |

| Rescue Service Phone |  |
| --- | --- |

SECTION 7 — RESCUE PLAN

| Non-Entry Rescue Method (preferred) | Emergency Contact / 911 Access Point |
| --- | --- |

PERMIT CANCELLATION / CLOSURE

| Reason for Cancellation or Completion | Cancelled / Closed By | Time |
| --- | --- | --- |

Cancelled permits must be retained for at least 1 year per 29 CFR 1910.146(e)(6) so that failures can be reviewed during the annual review of the confined space program.

This template is provided for educational purposes by EHS Community (ehscommunity.com). It does not constitute legal advice. Reference: 29 CFR 1910.146 — Permit-Required Confined Spaces.